POWERTRAIN SYSTEM ANALYSIS

eBAJA Electric Vehicle Powertrain

Motor → Chain/Sprocket → Gearbox → Output Shaft → Tyre

Analytical calculations, MATLAB model results and engineering checks

# 1. Objective

The objective is to develop an analytical and MATLAB-based model of the vehicle powertrain to determine how motor current and motor speed are transformed through the chain drive, gearbox and output shaft into tyre torque, tyre speed, vehicle speed and tractive force. The analysis also checks motor power consistency and tyre-road traction limits.

## Powertrain Configuration

Motor → 15T motor sprocket → 14T gearbox sprocket → 9:1 gearbox → output shaft → rear tyre/wheel

The output shaft is treated as a 1:1 speed connection. The vehicle is modeled as rear-wheel drive without a differential, using the stated drivetrain arrangement.

# 2. Complete Powertrain Input Sheet

| Parameter | Symbol | Value | Unit | Source / basis |
| --- | --- | --- | --- | --- |
| Rated motor power | P_motor,rated | 5 | kW | Motor datasheet |
| Peak motor power | P_motor,peak | 9 | kW | Motor datasheet; peak/short-duration |
| Rated motor torque | T_motor,rated | 15.5 | Nm | Motor datasheet |
| Maximum motor torque | T_motor,max | 90 | Nm | Motor datasheet |
| Rated motor speed | N_motor,rated | 4200 | RPM | Motor datasheet |
| Maximum motor speed | N_motor,max | 4300 | RPM | Upper end of stated 4100–4300 RPM range |
| Rated current | I_cont | 80 | A | Motor datasheet |
| Maximum current | I_max | 180 | A | Motor datasheet |
| Torque constant | K_t | 0.19375 | Nm/A | Derived from 15.5 Nm / 80 A; modeling assumption |
| Motor efficiency | η_motor | 97 | % | Motor datasheet |
| Motor-side sprocket | Z_motor | 15 | teeth | Given drivetrain data |
| Gearbox-side sprocket | Z_GB | 14 | teeth | Given drivetrain data |
| Chain reduction | R_chain | 0.9333 | ratio | Calculated |
| Gearbox reduction | R_GB | 9.00 | ratio | Given drivetrain data |
| Total reduction | R_total | 8.4000 | ratio | Calculated |
| Overall mechanical efficiency | η_mech | 77.5 | % | Assumed midpoint of 75–80% |
| Output shaft ratio | R_shaft | 1.00 | ratio | Assumed direct 1:1 shaft |
| Tyre rolling radius | R_tyre | 0.3048 | m | 24 in diameter / 2 |
| Vehicle mass incl. driver | M | 320 | kg | Given vehicle data |
| Driven-wheel load fraction | W_d fraction | 55 | % | Engineering assumption |
| Coefficient of friction | μ | 0.70 | — | Engineering assumption |

# 3. Analytical Calculation Sheet

## 3.1 Chain Reduction

R_chain = Z_GB / Z_motor

R_chain = 14 / 15 = 0.9333

Since the gearbox-side sprocket has fewer teeth than the motor-side sprocket, the chain stage is a slight overdrive (speed increase and torque decrease), not a reduction.

## 3.2 Total Reduction

R_total = R_chain × R_GB

R_total = 0.9333 × 9 = 8.4000:1

## 3.3 Gearbox Input and Tyre Speed

N_GB,in = N_motor / R_chain

At 4300 RPM: N_GB,in = 4300 / 0.9333 = 4607.14 RPM

N_tyre = N_motor / R_total

At 4300 RPM: N_tyre = 4300 / 8.4000 = 511.90 RPM

## 3.4 Motor Torque from Current

For the linear region, T_motor = K_t × I

K_t = 15.5 / 80 = 0.19375 Nm/A

Because the same linear constant cannot simultaneously reproduce 15.5 Nm at 80 A and 90 Nm at 180 A, the MATLAB model uses a two-segment torque-current approximation: linear to 80 A, followed by linear interpolation from 15.5 Nm at 80 A to 90 Nm at 180 A. This is an engineering modeling assumption rather than a manufacturer torque-current map.

## 3.5 Tyre Torque

T_tyre = T_motor × R_chain × R_GB × η_mech

At peak motor torque: T_tyre,max = 90 × 0.9333 × 9 × 0.775 = 585.90 Nm

## 3.6 Vehicle Speed

V = (N_tyre × 2πR_tyre / 60) × 3.6

At maximum motor speed: V_max,theoretical = 58.82 km/h

## 3.7 Tractive Force

F_tractive = T_tyre / R_tyre

At peak tyre torque: F_tractive,max = 585.90 / 0.3048 = 1922.24 N

## 3.8 Traction Limit

W_d = 0.55 × M × g

W_d = 0.55 × 320 × 9.81 = 1726.56 N

F_traction,max = μ × W_d

F_traction,max = 0.70 × 1726.56 = 1208.59 N

T_traction,max = F_traction,max × R_tyre

T_traction,max = 1208.59 × 0.3048 = 368.38 Nm

## 3.9 Power Consistency

P = T × ω, where ω = 2πN/60

Using the 5 kW continuous/rated power limit, the calculated constant-torque break speed is N_break = 3080.4 RPM, corresponding to approximately 42.14 km/h.

Above this speed, the MATLAB model reduces motor torque approximately as T = P/ω so that continuous motor power remains at 5 kW. The resulting current falls from approximately 80 A at the break point to about 57.3 A at 4300 RPM.

# 4. Final Results

| Result | Value | Unit |
| --- | --- | --- |
| Maximum motor torque | 90.00 | Nm |
| Maximum motor speed | 4300 | RPM |
| Motor maximum current | 180 | A |
| Motor sprocket teeth | 15 | teeth |
| Gearbox input sprocket teeth | 14 | teeth |
| Chain ratio | 0.9333 | ratio |
| Gearbox reduction ratio | 9.00 | ratio |
| Total reduction ratio | 8.4000 | ratio |
| Overall mechanical efficiency | 77.5 | % |
| Output shaft ratio | 1.00 | ratio |
| Maximum tyre torque (peak) | 585.90 | Nm |
| Maximum tyre RPM | 511.90 | RPM |
| Maximum theoretical vehicle speed | 58.82 | km/h |
| Maximum tractive force from peak tyre torque | 1922.24 | N |
| Traction-limited tyre torque | 368.38 | Nm |
| Continuous motor power used for main speed model | 5.00 | kW |

# 5. Operating Limits and Engineering Interpretation

The 5 kW rated power is used as the continuous operating basis of the speed/torque model.

The 9 kW motor rating is treated only as peak/short-duration capability and is not assumed to be continuously sustainable.

The 180 A current is the modeled maximum current limit.

The theoretical top speed is approximately 58.82 km/h at 4300 motor RPM and assumes no road load, aerodynamic drag or additional speed-dependent losses.

The peak calculated tyre torque (585.90 Nm) exceeds the estimated traction-limited tyre torque (368.38 Nm). Therefore, under the stated μ and driven-wheel-load assumptions, peak launch torque is traction-limited rather than fully usable at the tyre.

The actual vehicle speed under load can be lower than the theoretical value because of rolling resistance, aerodynamic drag, terrain, battery/controller limits, tyre slip and other losses.

# 6. MATLAB Model / Simulation

The MATLAB model contains editable motor, transmission, tyre and vehicle parameters and generates the required performance datasets. The model sweeps motor current and motor speed and calculates motor torque, tyre torque, tyre RPM, vehicle speed, tractive force, motor power and motor current.

The MATLAB script used for the analysis should be submitted with this report as a separate .m file.

## Modeling assumptions implemented

Chain + gearbox + output shaft mechanical efficiency = 77.5% overall.

Output shaft speed ratio = 1:1.

Tyre rolling radius = 0.3048 m.

Vehicle mass including driver = 320 kg.

Driven-wheel normal load = 55% of total vehicle weight.

Coefficient of friction μ = 0.7.

Torque-current model uses the derived Kt = 0.19375 Nm/A in the linear region and a two-segment approximation to the peak torque point.

5 kW is the continuous/rated power limit for the main torque-speed model.

# 7. Mandatory MATLAB Plots

Insert the generated MATLAB plots below/in the following order. The plots are not embedded in this document so that the final report can use the user's preferred plot formatting and resolution.

Graph 1: Motor Torque vs Motor Current

Graph 2: Tyre Torque vs Motor Current

Graph 3: Motor Speed vs Tyre Speed

Graph 4: Motor Speed vs Vehicle Speed

Graph 5: Tyre Torque vs Vehicle Speed

Graph 6: Motor Power vs Motor Speed

Graph 7: Tractive Force vs Vehicle Speed

Graph 8: Motor Current vs Vehicle Speed

# 8. Additional Combined MATLAB Plots

The following combined plots may be included after the mandatory plots as supplementary visualization:

Combined Figure 1: Motor Speed, Tyre Speed and Vehicle Speed

Combined Figure 2: Tyre Torque and Tractive Force

Combined Figure 3: Motor Current and Motor Power

# 9. Final Deliverables Checklist

| No. | Required deliverable | Status |
| --- | --- | --- |
| 1 | Complete powertrain input sheet with manufacturer references or clearly stated assumptions. | Included |
| 2 | Hand calculations / analytical calculation sheet for chain, gearbox, shaft and tyre stages. | Included |
| 3 | MATLAB code or Simulink model with editable parameters. | MATLAB model prepared separately |
| 4 | All mandatory graphs with correct units, labels, legends and operating limits. | Plots to be pasted in Section 7 |
| 5 | Gear-ratio comparison table for at least three candidate configurations. | Omitted from this report per current analysis scope |
| 6 | Traction-limit and power-consistency checks. | Included |
| 7 | Final recommendation of chain + gearbox ratio with engineering justification. | Current drivetrain documented; formal ratio comparison omitted per scope |
| 8 | One-page summary of maximum tyre torque, theoretical top speed and key operating constraints. | Included below |

# 10. One-Page Summary

The analyzed eBAJA powertrain consists of a 5 kW rated electric traction motor driving a 15-tooth sprocket, a 14-tooth gearbox sprocket, a 9:1 gearbox, a direct 1:1 output shaft and the rear tyre/wheel. The chain ratio is 0.9333, giving a total drivetrain ratio of 8.4000:1.

The motor has a rated torque of 15.5 Nm, peak torque of 90 Nm, rated current of 80 A, maximum current of 180 A and maximum modeled speed of 4300 RPM. The overall mechanical efficiency is assumed to be 77.5%.

At peak motor torque, the calculated tyre torque is approximately 585.9 Nm. However, using a 55% driven-wheel load and μ = 0.7, the estimated traction-limited tyre torque is only 368.4 Nm. This indicates that the peak drivetrain torque can exceed the available tyre-road traction under the stated assumptions.

Using the 4300 RPM motor speed limit, the calculated maximum tyre speed is 511.9 RPM and the maximum theoretical vehicle speed is 58.82 km/h. This is a theoretical unloaded value; actual speed will depend on vehicle resistance, terrain, controller limits, battery condition, tyre slip and other losses.

For the continuous operating model, 5 kW is used as the main motor power limit. The torque-speed model holds 15.5 Nm up to approximately 3080 RPM (42.1 km/h), after which torque decreases approximately inversely with speed to maintain 5 kW. The 9 kW rating is therefore treated as peak/short-duration capability.

Overall, the modeled drivetrain provides substantial launch torque, but peak torque utilization is expected to be traction-limited under the selected assumptions. The calculated top speed should be treated as a theoretical upper bound rather than a guaranteed loaded vehicle speed.

# 11. Assumptions to Declare

| Assumption | Value / statement |
| --- | --- |
| Chain efficiency | Included within 77.5% overall mechanical efficiency assumption. |
| Gearbox efficiency | Included within 77.5% overall mechanical efficiency assumption. |
| Output shaft efficiency | Included within 77.5% overall mechanical efficiency assumption; shaft ratio = 1:1. |
| Tyre rolling radius | 0.3048 m, based on 24-inch tyre diameter. |
| Coefficient of friction | μ = 0.7, engineering assumption. |
| Driven-wheel normal load | 55% of total vehicle weight, engineering assumption. |
| Motor torque-current model | Kt derived from rated point; two-segment approximation used to reach peak torque. |
| Motor/controller power limit | 5 kW continuous/rated basis; 9 kW treated as peak capability. |
| Other losses | Not explicitly modeled beyond the combined mechanical efficiency. |

# 12. Conclusion

The MATLAB-based analytical model gives a peak calculated tyre torque of 585.90 Nm and a theoretical maximum vehicle speed of 58.82 km/h for the specified 15T/14T chain drive and 9:1 gearbox. The traction check gives a tyre torque limit of 368.38 Nm under the stated μ and driven-load assumptions, so peak drivetrain torque is above the available traction limit. The continuous performance model is based on the 5 kW rated motor power, while the 9 kW rating is treated as a short-duration peak capability.